2Kx2K RGB Drive Board

For Use with eMagin 2Kx2K OLED Microdisplays

USER’S MANUAL

Version 2.0

Table of Contents

1.	Introduction	1

2.	FEATURES	1

2.1.	Software Features	1

3.	SYSTEM REQUIREMENTS & SPECIFICATIONS	1

3.1.	System Requirements	1

3.2.	2KX2K RGB Drive Board.	2

4.	INTERFACE CONNECTIONS & SETUP	3

4.1.	Setup Flow Chart	3

4.2.	Connect Display to the RGB Drive Board	4

4.3.	Setup PC for Proper Video Output	4

4.4.	Power Up	5

4.5.	Power Down	5

4.6.	Brightness	5

5.	Using the 2KX2K RGB DRIVE BOARD Software	5

5.1.	Serial Interface Command Set	5

5.2.	2KX2K Design Reference Board Software Utility	5

5.2.1.	Hardware Protocol	7

5.2.2.	eMagin Software Command Set	7

5.3.	Using Hyperterminal With the Design Reference Board	8

5.3.1.	ASCII Table of Commands	10

5.4.	Downloading New Firmware Versions to the RGB Drive Board	11

5.4.1.	Using the Firmware Download Utility	11

6.	Advanced User Information	12

6.1.	Microcontroller	12

6.2.	I2C bus	12

6.3.	SPI bus	12

6.4.	System Hardware Overview	12

6.5.	Voltages	13

6.6.	Bias Control	13

6.7.	Gamma	13

6.7.1.	Gamma Coefficients	14

6.7.2.	Preset Gamma Table descriptions	14

6.7.3.	Using “Update LUT” Button for Auto-Gamma Correction	15

6.7.4.	The “Read” Button	15

6.7.5.	System Gamma	15

7.	Revision History	15

List of Tables

TABLE 5-1 COMMAND SET LIST FOR ADDRESSING THE MICRODISPLAY	7

TABLE 5-2 TABLE OF ASCII CHARACTER CODES	10

List of Figures

Figure 3-1  2Kx2K RGB Drive Board Rev. 1	2

Figure 4-1 Display Carrier Board ZIF Connection Orientation	4

Figure 5-1 Example microdisplay register value write using included software	8

Figure 5-2 Example microdisplay register value write using Hyperterminal	9

Figure 5-3 Firmware download utility	11

# Introduction

The 2Kx2K RGB Drive Board, provides the user with a highly compact, portable way of operating an eMagin 2KX2K OLED Microdisplay.  This product was designed to deliver a complete tool for developers to evaluate and integrate eMagin 2Kx2K microdisplays into new products. The included software package provides access to the microdisplay’s on-board register settings from any Windows-based PC through a serial port.

# FEATURES

- DVI Input

- USB (serial) interface allows access to microdisplay registers

- Supporting Software (Windows)

- ON/OFF power switch

## Software Features

- Read/write capabilities allow adjustments of microdisplay register settings to fine-tune image characteristics

- Software register control over the cathode voltage (Vcommon) input

- Software register control over the microdisplay’s brightness

- Download and install new firmware files into your 2KX2K RGB Drive Board for easy upgrades and expanded functionality

- Ability to read the microdisplay’s temperature

- Save feature stores custom register settings for convenience

# SYSTEM REQUIREMENTS & SPECIFICATIONS

## System Requirements

- For digital RGB inputs: A PC capable of producing a digital video output compliant with the DVI standard.

- Support software requires a Windows PC with a USB serial port

## 2KX2K RGB Drive Board.

The 2Kx2K RGB Drive Board is shown in figure 3-1 below. The minimum requirements for displaying video on the 2KX2K RGB Drive Board are; a 9v power supply, a USB cable connected to the USB connector, a digital video source connected to the HDMI connector and an 2Kx2K OLED connected to J5 and J6 connectors.

Figure 3-1  2KX2K RGB Drive Board

# INTERFACE CONNECTIONS & SETUP

## Setup Flow Chart

SET POWER

SWITCH TO OFF

2Kx2K DRK

SETUP

CONNECT THE

VIDEO SOURCE

TO THE DRK

INSTALL

MICRODISPLAY

IN SOCKET

SETUP VIDEO

SOURCE TO

2048x2048

0

TURN POWER

SWITCH TO ON

CONNECT THE

USB SERIAL PORT

TO THE DRK

CONNECT

USB Cable

TO THE DRK

## Connect Display to the RGB Drive Board

The microdisplay connects to the RGB Drive Board via J1 a 36-pin board-to-board connector and a J2 a 40-pin connector. The Display Carrier board is shown in figure 4-1 below.

Note: The 2KX2K OLED can be damaged if it is not connected properly.

Figure 4-2 Display Carrier Board

Lift the display by its sides and take care not to press on the active area or leave fingerprint marks on it.  Insert the display into the RGB Drive Board.

Warning

The gray material at each side of the microdisplay assembly is relatively soft. It protects the wires connecting the microdisplay die to the carrier board.

DO NOT PRESS ON THESE GRAY AREAS to install or remove the microdisplay from the drive board. This will result in permanent and un-repairable damage to the microdisplay

Refer to the view below to correctly identify these areas and avoid applying any pressure to them

Figure 3: Microdisplay Handling

## Setup PC for Proper Video Output

- The default compatible resolution is 2KX2K (2048x2048) with a refresh rate of 60 Hz. If you are using this input set the video resolution for 2048x2048 in your PC’s display properties.

- Ensure that the refresh rate to set to 60 Hz, or other supported refresh rate per the display specification.

- If you are using a laptop PC, you may need to export the video signal to an external monitor. This is usually accomplished through a keypress including the Fn key + a designated function key. See your PC’s operation instructions for more information.

## Power Up

- Set the power switch to the ON position.

## Power Down

- Set the power switch to the OFF position.

## Brightness

eMagin strongly recommends that you drive the microdisplay at the minimum luminance necessary for your application. This will extend the lifetime of the display to its maximum possible lifetime. As OLED microdisplays are emissive devices, driving the microdisplay at high bias levels will decrease its overall lifetime.

# Using the 2KX2K RGB DRIVE BOARD Software

The 2Kx2K RGB Drive Board includes a support software suite with the following functionality.

- Perform software microdisplay brightness adjustments

- Perform software adjustments of the Vcommon input to the microdisplay

- Read the microdisplay’s temperature

- Download and install new versions of the 2Kx2K RGB Drive Board firmware to update or provide new functionality using a USB connection

- Read/write register values to the microdisplay to control various characteristics (see your microdisplay’s User Specification for more information)

## Serial Interface Command Set

The 2Kx2K RGB Drive Board can be controlled by sending commands and data using the included USB cable. The software package includes a file called New_2Kx2K_RGB_SW.exe developed for this purpose.

## 2KX2K Design Reference Board Software Utility

- Connect the USB cable to the PC (Port 1) and to the USB connector on the 2KX2K RGB Drive Board

- Connect the video source to the 2KX2K Design Reference Board

- Turn on the 2KX2K RGB Drive Board

- Start the 2KX2K_SW_V1_1.exe application and a screen should appear like that shown below:

Figure 5-1 2KX2K Design Reference Board Software Utility Showing The Oled Registers

- The application communicates with the PIC on the 2KX2K RGB Drive Board and reads the initial status of the “OLED” and “VPG” shutdown pins.

- The read/write buttons shown in the “OLED Registers” panel will read/write the register settings in their corresponding boxes from/to the OLED microdisplay.

- The read/write buttons displayed in the “GAMMA” panel will read/write the values used in generating a Gamma table that resides in the display.

Figure 5.2 2KX2K RGB Drive Board Software Utility Showing the Gamma Tab

Direct control of the 2KX2K RGB Drive Board can also be implemented into a customer specific application using the following hardware protocol and command set.

### Hardware Protocol

- bit, no parity

- 9600 baud

- No hardware handshake

### eMagin Software Command Set

TABLE 5-1 COMMAND SET LIST FOR ADDRESSING THE MICRODISPLAY

REGISTERS

| Command | Description |
| --- | --- |
| O | Select the OLED device Usage: O |
| E | Read the EEPROM device Usage: E Note: The E command is preceded by a HX command |
| J | Send a data value to the 2KX2K Design Reference Board Usage: JX where X = 0 to 255 |
| H | Send a register value to the 2KX2K Design Reference Board Usage: HX where X = 0 to 255 |
| W | Write a register Usage: W Note: The W command is preceded by a HX JY command |
| G | Change the Gamma Table Usage: GX where X = 0 to 3 |
| P | Powerdown a device on the 2KX2K Design Reference Board Usage: PX where X = 0 to 7 |
| Q | Returns the value at the A2D pin. Usage: Q Returns two values(high byte, low byte) |
| R | Read a register Usage: R Note: The R command is preceded by a HX JY command |
| S | Returns the status of a device on the 2KX2K Design Reference Board Usage: SX where X = 0 to 7 |
| U | Update the Gamma lookup table in the FPGA. Usage: U |

Note: The commands and values are typically generated from a control program. The values are sent in binary format to the RGB Drive Board microcontroller, which will process these natively in binary. Below is an example write instance using the provided software:

In order to write the decimal value 81 to register 3, the following commands need to be issued: G3D81W

No space is required between the commands

Figure 5-4 Example microdisplay register value write using included software

## Using Hyperterminal With the Design Reference Board

If you will use Hyperterminal to control the RGB Drive Board please note that Hyperterminal converts all input data to ASCII characters. Therefore, the X value mentioned in the table above needs first to be converted to its ASCII equivalent in order to be recognized by the RGB Drive Board firmware. Received information will also be displayed as ASCII code. An ASCII to decimal (or hexadecimal) conversion must be performed to read the correct values sent via the serial interface. We have provided an example write instance and ASCII character table for your reference.

To write the decimal value 81 to register 3, the following commands need to be issued:

G CTRL-C D Q W

Ctrl+C (pressing the Ctrl and C key simultaneously) is the ASCII equivalent of decimal 3

Note: No space is required between the commands. The spacing above is for clarity only.

Figure 5-5 Example microdisplay register value write using Hyperterminal

### ASCII Table of Commands

TABLE 5-2 TABLE OF ASCII CHARACTER CODES

| Non-Printing Characters |  |  |  |  |  | Printing Characters |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Ctrl char | Dec | Hex | Char |  | Dec | Hex | Char |  | Dec | Hex | Char |  | Dec | Hex | Char |
| Null | ctrl-@ | 0 | 00 | NUL |  | 32 | 20 | Space |  | 64 | 40 | @ |  | 96 | 60 | ` |
| start of heading | ctrl-A | 1 | 01 | SOH |  | 33 | 21 | ! |  | 65 | 41 | A |  | 97 | 61 | a |
| start of text | ctrl-B | 2 | 02 | STX |  | 34 | 22 | " |  | 66 | 42 | B |  | 98 | 62 | b |
| end of text | ctrl-C | 3 | 03 | ETX |  | 35 | 23 | # |  | 67 | 43 | C |  | 99 | 63 | c |
| end of xmit | ctrl-D | 4 | 04 | EOT |  | 36 | 24 | $ |  | 68 | 44 | D |  | 100 | 64 | d |
| Enquiry | ctrl-E | 5 | 05 | ENQ |  | 37 | 25 | % |  | 69 | 45 | E |  | 101 | 65 | e |
| Acknowledge | ctrl-F | 6 | 06 | ACK |  | 38 | 26 | & |  | 70 | 46 | F |  | 102 | 66 | f |
| Bell | ctrl-G | 7 | 07 | BEL |  | 39 | 27 | ' |  | 71 | 47 | G |  | 103 | 67 | g |
| Backspace | ctrl-H | 8 | 08 | BS |  | 40 | 28 | ( |  | 72 | 48 | H |  | 104 | 68 | h |
| horizontal tab | ctrl-I | 9 | 09 | HT |  | 41 | 29 | ) |  | 73 | 49 | I |  | 105 | 69 | i |
| line feed | ctrl-J | 10 | 0A | LF |  | 42 | 2A | * |  | 74 | 4A | J |  | 106 | 6A | j |
| vertical tab | ctrl-K | 11 | 0B | VT |  | 43 | 2B | + |  | 75 | 4B | K |  | 107 | 6B | k |
| form feed | ctrl-L | 12 | 0C | FF |  | 44 | 2C | , |  | 76 | 4C | L |  | 108 | 6C | l |
| carriage feed | ctrl-M | 13 | 0D | CR |  | 45 | 2D | - |  | 77 | 4D | M |  | 109 | 6D | m |
| shift out | ctrl-N | 14 | 0E | SO |  | 46 | 2E | . |  | 78 | 4E | N |  | 110 | 6E | n |
| shift in | ctrl-O | 15 | 0F | SI |  | 47 | 2F | / |  | 79 | 4F | O |  | 111 | 6F | o |
| data line escape | ctrl-P | 16 | 10 | DLE |  | 48 | 30 | 0 |  | 80 | 50 | P |  | 112 | 70 | p |
| device control 1 | ctrl-Q | 17 | 11 | DC1 |  | 49 | 31 | 1 |  | 81 | 51 | Q |  | 113 | 71 | q |
| device control 2 | ctrl-R | 18 | 12 | DC2 |  | 50 | 32 | 2 |  | 82 | 52 | R |  | 114 | 72 | r |
| device control 3 | ctrl-S | 19 | 13 | DC3 |  | 51 | 33 | 3 |  | 83 | 53 | S |  | 115 | 73 | s |
| device control 4 | ctrl-T | 20 | 14 | DC4 |  | 52 | 34 | 4 |  | 84 | 54 | T |  | 116 | 74 | t |
| neg acknowledge | ctrl-U | 21 | 15 | NAK |  | 53 | 35 | 5 |  | 85 | 55 | U |  | 117 | 75 | u |
| synchronous idel | ctrl-V | 22 | 16 | SYN |  | 54 | 36 | 6 |  | 86 | 56 | V |  | 118 | 76 | v |
| end of xmit block | ctrl-W | 23 | 17 | ETB |  | 55 | 37 | 7 |  | 87 | 57 | W |  | 119 | 77 | w |
| cancel | ctrl-X | 24 | 18 | CAN |  | 56 | 38 | 8 |  | 88 | 58 | X |  | 120 | 78 | x |
| end of medium | ctrl-Y | 25 | 19 | EM |  | 57 | 39 | 9 |  | 89 | 59 | Y |  | 121 | 79 | y |
| substitute | ctrl-Z | 26 | 1A | SUB |  | 58 | 3A | : |  | 90 | 5A | Z |  | 122 | 7A | z |
| escape | ctrl-[ | 27 | 1B | ESC |  | 59 | 3B | ; |  | 91 | 5B | [ |  | 123 | 7B | { |
| file separator | ctrl-\ | 28 | 1C | FS |  | 60 | 3C | < |  | 92 | 5C | \ |  | 124 | 7C | ¦ |
| group separator | ctrl-] | 29 | 1D | GS |  | 61 | 3D | = |  | 93 | 5D | ] |  | 125 | 7D | } |
| record separator | ctrl-^ | 30 | 1E | RS |  | 62 | 3E | > |  | 94 | 5E | ^ |  | 126 | 7E | ~ |
| unit separator | ctrl-_ | 31 | 1F | US |  | 63 | 3F | ? |  | 95 | 5F | _ |  | 127 | 7F | DEL |

## Downloading New Firmware Versions to the RGB Drive Board

The ability to download new versions of the firmware ensures that you will have the latest functionality without having to send your 2KX2K RGB Drive Board for reprogramming. A utility that downloads and installs new firmware versions is included in the software package.

### Using the Firmware Download Utility

Firmware files can be downloaded as hex files. Before attempting to download and install new firmware versions make sure that you have received a firmware hex file from an eMagin source.

To load your firmware files, follow the following steps:

- Connect the USB cable to the PC and to the Mini USB connector on the 2KX2K RGB Drive Board

- Connect your video source to the 2KX2K RGB Drive Board.

- Start the New_2KX2K_RGB_SW.exe application. Select the “Bootloader” tab at the top of the form. A window should appear like that below:

Figure 5-6 Firmware download utility

- Click the “Program” button to find the hex file you wish to install. Note: The power on the 2KX2K RGB Drive Board should be off while setting up your download. Only turn on the power to the 2KX2K RGB Drive Board when you are ready to write your new firmware to the PIC.

- When you turn on the 2KX2K RGB Drive Board the hex file should begin to upload. You may have to toggle the switch several times.

- When the upload is complete, the 2KX2K RGB Drive Board will run.

# Advanced User Information

## Microcontroller

The 2KX2K RGB Drive Board utilizes a Microchip PIC18LF46K22 microcontroller.  This is a CMOS Flash microcontroller in a 40 pin package.  Provisions are on the circuit board for in circuit reprogramming but are not accessible without opening the case. Please refer to the Microchip PIC18LF46K22 datasheet available from Microchip Technology Inc. for additional information about the microcontroller and programming.

The main function of the microcontroller is to communicate with the OLED display. All communication is over the internal I2C or SPI bus and the microcontroller acts as the bus master.  On power up the registers of the OLED are initialized with the default settings programmed into the microcontroller.  After initialization the microcontroller scans for user input and monitors the OLED.

## I2C bus

The microcontroller is the bus master and communication occurs at the standard 100KHz clock rate.  The microcontroller and the OLED all operate from 2.5VDC so the I2C bus must also operate using 2.5V logic levels.  The maximum speed allowed for I2C bus communication is 400KHz.  Please refer to the Philips I2C bus specification available on the Philips website for detailed information.

## SPI bus

The microcontroller is the bus master and communication occurs at the standard 100KHz clock rate.  The microcontroller and the OLED all operate from 2.5VDC so the SPI bus must also operate using 2.5V logic levels.  The maximum speed allowed for SPI bus communication is 400KHz.

## System Hardware Overview

Figure 6-1 below is a block diagram of the main functions on the 2KX2K RGB Drive Board.

Figure 6-1 System level Block Diagram

## Voltages

The power supply section of the 2KX2K RGB Drive Board consists of several voltage regulators, some of which are adjustable with potentiometers. Table 6-2 below describes the voltages and there purpose.

| Voltage | Range | Typical | Purpose |
| --- | --- | --- | --- |
| Vinput | +5.0 Volts | +5.0Volts | Main Supply |
| V5 | 5Volts ±10% (fixed) | 5 Volts | LEDS, DAC |
| V3.3 | 3.3Volts ±10% (fixed) | 3.3 Volts | DVI, RS232 |
| V2.5 | 2.5Volts ±10% (fixed) | 2.5 Volts | OLED |
| VAN | 5.0V (adjustable with pot) | 5.0 Volts | OLED Anode |
| VPG | -1.5V (adjustable with pot) | -1.5V | OLED Bias |
| VDD | 2.5V ±10% (fixed) | 2.5V | OLED |

Table 6-2 shows the voltage and current relationship at power-on.

## Bias Control

BIASN: Normal board – BIASN=2 gives the best results.

## Gamma

To compensate for the non-linear dependency of luminance on the voltage of the OLED diode, the R, G and B signals include internal gamma correction to linearize the pixel response as a function of the input video signal.

The gamma correction consists of a 17 values whose parameters are set via registers in the “Gamma” panel on the windows software. All three color channels have the same PWL shape.

There are multiple ways to set the Gamma using the windows software. The Gamma section of the software is shown below.

Figure 6-5 The Gamma Section of the Software

### Gamma Coefficients

The 9 boxes in the Gamma section represent the 17 values that define the gamma curve.  These 9 values are used to calculate the 256 element look-up-table (LUT) that is required to transform input video data into a gamma-corrected data signal for the microdisplay input port. The “Read” button will retrieve the current 9 values from the microcontroller. The user can change any of the 9 values and see the resulting gamma corrected video by pressing the “Write” button.

### Preset Gamma Table descriptions

There is 1 default Gamma table programmed into the Microcontroller firmware and can be selected from the windows software. Pressing the “Default” button in the “Gamma” section of the software loads the default gamma values for all 3 colors. Pressing the “Write” button loads the gamma values into the display.

### Using “Update LUT” Button for Auto-Gamma Correction

The software allows for the immediate update of the gamma tables with the push of a button, “Update LUT”. This button, located in the center of the software form, tells the firmware to calculate the Gamma coefficients using the VGN signal provided by the 2Kx2K microdisplay. This feature allows the display gamma to be automatically adjusted for any operating conditions of temperature and brightness. The firmware calculates the 9 Gamma coefficients and then the full 256 value lookup table.

### The “Read” Button

The software allows for the reading of the full 9 values, for each of the 3 colors, using this button.

### System Gamma

The software allows for the overall System Gamma to be set between 0.5 and 2.5 when using the Auto-Gamma feature. This System Gamma can be used to compensate for the gamma of the input video. For example, if the source video is gamma corrected to 2 then setting the System Gamma to 1.9 and pressing the “Set” button and then the “Update LUT” button will give a better overall gamma response. This does not mean that there is a 1 to 1 relationship between input gamma and the System Gamma.

# Revision History

| Revision Level | Date | Description |
| --- | --- | --- |
| A | 09/22/17 | Initial Release. |